# Vision-Guided Robotic Sorting System

## Technical Operations Manual

This document provides a structured, professionally formatted operating manual for the Vision‑Guided Robotic Sorting System, covering hardware setup, controller configuration, ROS2 launch workflow, and system operation. It is rewritten for clarity, consistency, and technical readability.

## 1. Hardware Configuration

- Connect Host PC (IP 192.168.2.1) to UR Controller (IP 192.168.2.2) using Ethernet.

- Connect Intel RealSense D432 depth camera to PC via USB.

- ROS2 workspace located at ~/humble_ws/.

- UR code stored under ~/humble_ws/src/my_ur_description/.

## 2. UR Controller Setup (Polyscope X)

- Set network mode to Static with IP 192.168.2.2 and subnet 255.255.255.0.

- Ensure required services/interfaces: RTDE, Primary/Secondary Clients, Interpreter Mode, Modbus TCP Server, ROS2.

- Verify External Control URCap configured to PC IP 192.168.2.1 port 50002.

- Run External Control program before starting motion control.

## 3. PC Preparation

- Source ROS environments: /opt/ros/humble/setup.bash and ~/humble_ws/install/setup.bash.

- Set ROS_DOMAIN_ID=43 in each terminal.

- Up to 9 terminals required for full system operation.

## 4. ROS2 Launch Workflow (Terminal-by-Terminal)

- Terminal 1: Run grasp_perception node.

- Terminal 2: Run camera parameter control.

- Terminal 3: Launch ur_robot_driver for UR12e.

- Terminal 4: Verify controllers and launch MoveIt2.

- Terminal 5: Load planning scene and test robot motion.

- Terminal 6: Start SOCAT gripper bridge.

- Terminal 7: Validate /tmp/ttyUR and test gripper.

- Terminal 8: Start gripper server node.

- Terminal 9: Run sorting brain logic.

## 5. Sorting Modes

- Single pick using position control.

- Single pick using force control.

- Repeated sorting cycle (position-based).

- Repeated sorting cycle (force-based).

## 6. Shutdown Procedure

- Use CTRL-C in active terminals to stop processes.

- Send robot to home pose using home.py.