# Vision-Guided Robotic Sorting System

## Technical Operations Manual

This is the polished, professionally structured technical version of the operating instructions, prepared for engineering documentation, audits, or handover packages.

# 1. System Overview

The Vision‑Guided Robotic Sorting System integrates:

- A UR12e collaborative robot

- Intel RealSense D432 depth camera

- ROS 2 Humble middleware

- MoveIt2 motion planning framework

- Robotiq gripper (serial bridge via SOCAT)

The system performs automated object detection, grasp planning, sorting, and force‑controlled placement.

# 2. Hardware Configuration

### 2.1 Connectivity

- Host PC (IP 192.168.2.1) → UR Controller (IP 192.168.2.2) via Ethernet

- Intel D432 depth camera → Host PC via USB

### 2.2 ROS2 Workspace Structure

1 ~/humble_ws/ 2 ~/humble_ws/src/my_ur_description/ 3 ~/humble_ws/src/my_ur_description/scripts/ 4

# 3. UR Controller (Polyscope X) Configuration

### 3.1 Network Setup

Navigate: Settings → Network

- Mode: Static

- IP Address: 192.168.2.2

- Subnet Mask: 255.255.255.0

### 3.2 Services Required

Navigate: Settings → Services and confirm:

- Primary & Secondary Client Interfaces

- Real-Time Client

- RTDE (Real-Time Data Exchange)

- Interpreter Mode Socket

- Modbus TCP Server

- ROS2 Support

### 3.3 URCaps Verification

Navigate: Application

- External Control URCap → Host: 192.168.2.1:50002

- Tool Communication Forwarder URCap → Socat Server: Running

### 3.4 Program Requirements

Ensure program contains:

1 set_tool_voltage(24) 2 set_tool_communication(True, 115200, 0, 1, 1.5, 3.5) 3

Also ensure External Control Program (cached from PC) is available.

# 4. PC Preparation

### 4.1 ROS Environment Setup

Each terminal must include:

1 source /opt/ros/humble/setup.bash 2 source ~/humble_ws/install/setup.bash 3 export ROS_DOMAIN_ID=43 4

### 4.2 Terminal Requirements

A full system run requires 6–9 open terminals.

# 5. Launch Workflow (Terminal-by-Terminal)

### Terminal 1 — Perception Node

Runs grasp detection:

1 ros2 run grasp_perception grasp_node5 2

### Terminal 2 — Camera Parameter Control

Launch RealSense parameter or tuning nodes.

### Terminal 3 — UR Robot Driver

1 ros2 launch ur_robot_driver ur_control.launch.py \ 2 ur_type:=ur12e \ 3 robot_ip:=192.168.2.2 \ 4 use_fake_hardware:=false \ 5 kinematics_params:=./src/my_ur_description/config/ur12e_calibration.yaml \ 6 initial_joint_controller:=scaled_joint_trajectory_controller 7

### Terminal 4 — Controller Validation + MoveIt2

Check controllers:

1 ros2 control list_controllers 2

Activate if needed:

1 ros2 control switch_controllers --activate scaled_joint_trajectory_controller --deactivate joint_trajectory_controller 2

Launch MoveIt2:

1 ros2 launch ur_moveit_config ur_moveit.launch.py \ 2 ur_type:=ur12e \ 3 launch_rviz:=true 4

### Terminal 5 — Scene Setup & Motion Tests

1 cd ~/humble_ws/src/my_ur_description/scripts 2 python3 setup_moveit_scene2.py 3 python3 move_plan_xyz_theta.py 4 python3 home.py 5

### Terminal 6 — SOCAT Gripper Bridge

1 while true; do 2 sudo killall -9 socat 2>/dev/null 3 socat pty,link=/tmp/ttyUR,raw,echo=0,waitslave tcp:192.168.2.2:54321,nodelay 4 sleep 1 5 done 6

### Terminal 7 — Gripper Verification

1 ls -l /tmp/ttyUR 2 sudo chmod 666 /tmp/ttyUR 3 python3 test_gripper_final_fixed_v3.py 4 python3 careful_gripper_test.py 5 python3 force_release2.py 6

### Terminal 8 — Gripper Server Node

1 python3 robotiq_gripper_server.py 2 # or 3 python3 robotiq_gripper_server2.py 4

### Terminal 9 — Brain Logic

Single pick:

1 python3 vision_guided_sorting_brain_once_gripper_client.py 2

Force-control pick:

1 python3 vision_guided_sorting_brain_once_gripper_client_force_contact.py 2

Repeated cycle:

1 ./run_picker_position.sh 2 ./run_picker_force.sh 3

# 6. Shutdown Procedure

- Stop active processes using CTRL‑C

- Return robot to home position:

1 python3 home.py 2

If you'd like, I can also produce:

- A fully illustrated PDF

- A laminated-style operator quick card

- A sequence flowchart for wall posters

- A troubleshooting appendix

Just tell me!