Full Instructions and Procedure for Operating the Vision Guided Robotic Sorting System

(this is a many-terminal-version, we may put some processes on background, but then we may have less control e.g. restarting a process by just pressing ctrl-c on the terminal and re-run it, and seemingly sometimes less robust)

(Green fonts are for setting up the first time)

Hardware Connection

Host PC (192.168.2.1) is connected directly to the UR controller (192.168.2.2) with an Ethernet cable

Intel Depth Camera D432 is connected to the host PC with a USB cable

The main ROS2 workspace is:

~/humble_ws/

All UR related source codes are in the folder:

~/humble_ws/src/my_ur_description

All UR Python scripts are stored in the folder:

~/humble_ws/src/my_ur_description/scripts

Software Stack:

On UR Teach Pendant (Polyscope X) :

Switched on UR Controller (take a long time)

Power on the Robot

Unlock the Robot

Go to Settings-Network:

Ensure Static, IP Address 192.168.2.2, Subnet Mask 255.255.255.0

Go to Settings-Services:

Ensure Primary Client interface, Secondary Client interface, Real-Time Client interface, Real-Time Data Exchange (RDTE), Interpreter Mode Socket, Modbus TCP Server, ROS2 are there.

Go to Application:

Ensure No robotiq-appliation urcap is missing

External Control urcap is there and with 192.168.2.1:50002 as its computer host IP address and Port

Tool Communication Forwarder urcap is there with Socat Server running

Go to Program:

Ensure script of set_tool_voltage(24) and set_tool_communication(True, 115200, 0, 1, 1.5, 3.5) is there.

Ensure External Control Program: Cached from 192.168.2.1:50002 is there.

On Host PC (Ubuntu 22.04 with ROS2 Humble, Moveit2, Yolov8, ..)

Switched on the PC

Login with password

Open Terminal Windows (For robust operation, we may need to open up to 9 terminal windows!)

cat .bashrc   (check if the following lines are there, if no, you may need to run them at each terminals)

source /opt/ros/humble/setup.bash

source ~/humble_ws/install/setup.bash

export ROS_DOMAIN_ID=43     (just a randomly chosen number ensuring it is unique among ROS users there)

Terminal 1:

Eye and Percepton:  Grasp Perception Node:

ros2 run grasp_perception grasp_node5

Terminal 2:

Eye and Percepton:  Camera Parameter Control

Terminal 3:

Hand and Motion Planning: UR driver

ros2 launch ur_robot_driver ur_control.launch.py \

ur_type:=ur12e \

robot_ip:=192.168.2.2 \

use_fake_hardware:=false \

kinematics_params:=./src/my_ur_description/config/ur12e_calibration.yaml \

initial_joint_controller:=scaled_joint_trajectory_controller

A Rviz window with a robot reflecting the real physical one should pop up.

Terminal 4

Ensure scaled_joint_trajectory_controller, joint_state_broadcaster, force_torque_sensor_broadcaster are there and are active

ros2 control list_controllers

If say scaled_joint_trajectory_controller is not active, use the following command to activate it:

ros2 control switch_controllers \

--activate scaled_joint_trajectory_controller \

--deactivate joint_trajectory_controller

Hand: Run the External Control Program on UR Controller (Polyscope X)

Go to Program

Tap the line with External Control Program

Press Update program

Press the play button

Terminal 4:

Hand and Motion Planning: Moveit2

ros2 launch ur_moveit_config ur_moveit.launch.py \

ur_type:=ur12e \

launch_rviz:=true

Another Rviz window with an orange robot should pop up.

On this Moveit2 Rviz, you should be able to plan motion and execute the motion now.

Terminal 5:

Hand and Motion Planning: Setting up the Scene (table, virtual wall for collision avoidance planning) and Testing some simple motion

cd /humble_ws/src/my_ur_description/scripts

python3 ~/humble_ws/src/my_ur_description/scripts/setup_moveit_scene2.py

Some simple motions: (For safety, you may want to hold the Teach Pendant and prepare to press the emergency button whenever unexpected motion is seen.)

python3 python3 move_plan_xyz_theta.py

Go home:

python3 home.py

Terminal 6:

Gripper: SOCAT Gripper Bridge (the most complicated part, and not very stable)

while true; do

echo 'Reviving Bridge...'

sudo killall -9 socat 2>/dev/null

socat pty,link=/tmp/ttyUR,raw,echo=0,waitslave tcp:192.168.2.2:54321,nodelay

sleep 1

done

(need admin password)

Terminal 7:

Gripper: Check SOCAT Gripper Bridge

ls -l /tmp/ttyUR

Expected output:

…    /tmp/ttyUR -> /dev/pts/11

Ensure Permission:

sudo chmod 666 /tmp/ttyUR

(need admin password)

Check Gripper Operation:

cd /humble_ws/src/my_ur_description/scripts

python3 test_gripper_final_fixed_v3.py

python3 careful_gripper_test.py

Gripper-Force-Release:

python3 force_release2.py

Terminal 8:

Gripper: Server_Node

cd /humble_ws/src/my_ur_description/scripts

python3 robotiq_gripper_server.py

or

python3 robotiq_gripper_server2.py

Check if no answer from the gripper, or you may need to run the following commands and restart the gripper server node till the server node is online and communicate well with the gripper:

Terminal 7: python3 careful_gripper_test.py

Terminal 8: python3  robotiq_gripper_server2.py

Terminal 5 or Terminal 9 :

Brain :

Look and Pick Once with Position Control Only:

python3 vision_guided_sorting_brain_once_gripper_client.py

Look and Pick Once with Force Control:

cd /humble_ws/src/my_ur_description/scripts

python3 vision_guided_sorting_brain_once_gripper_client_force_contact.py

Repeated Sorting with Position Control only:

./run_picker_position.sh

(need admin password)

Repeated Sorting with Force Control:

./run_picker_force.sh

(need admin password)

To stop, select the terminal that run the script, press CTRL-C several times, till the prompt appears again.

To go back home, run:

python3 home.py

Finally, to exit:  power off the robot first, and then pressing CTRL-C on each terminals having process running or just directly shut down and power off the PC.